ASYNCHRONOUS FIFO — VERIFICATION PLAN

Author: Trần Hồ Khánh Tân

DUT: asyn_fifo.v  |  Reference: Design_Spec_Final.pdf

Note : wr_bin ( address use binary ), wr_ptr( use gray code)

| Number | Name block | Feature | Verification item | Test Method | Pass condition |
| --- | --- | --- | --- | --- | --- |
| 1 | Reset | Independent asynchronous reset | - wr_rst_n, rd_rst_n, wr_bin,rd_bin, wr_full, rd_empty, wr_ptr,rd_ptr | - Put the wr_rst_n = 0 and rd_rst_n = 1. After that, put wr_rst_n = 1 and rd_rst_n = 0. | - When wr_rst_n = 0, the wr_bin and wr_ptr goes Low ( = 0) immediate and wr_full = 0. Beside that, when rd_rst_n = 0, the rd_bin and rd_ptr also goes Low ( = 0 ) and rd_empty = 1. |
| 2 | Write | Single Write | - wr_inc, wdata, wr_bin, wr_full, write_accept | - Put the wr_rst_n = 1 back and wr_inc = 1 and write the wdata value in satisfied wr_bin( wr_address) when wr_full = 0 | - write_accept = 1 because ( wr_inc && !wr_full) and wr_full still in Low ( = 0) - wdata write correct value in the wr_bin ( address) - wr_bin increase 1 binary value - wr_ptr increase 1 value |
| 3 | Write | Write until full from empty | - wr_full, wr_bin, wr_ptr | - Put the wr_rst_n = 0 and then put wr_rst_n = 1 back . - Assert wr_inc = 1 continuous exact 8 times ( FIFO_DEPTH = 8) when rd_inc = 0 and write 8 wdata value in 8 address | - All 8 write operations have been write all successful and be accepted - wr_full go High ( = 1) after the 8th write 1 clock cycles - wr_ptr go from 0000 to 1100. - wr_bin go from 0000 to 1000. |
| 4 | Write | Write when full ( overflow protection) | - wr_full, wr_ptr, wr_bin, wdata, | - When wr_full = 1, asserts wr_inc = 1 for about 2 - 3 more cycles with wdata value | - wr_full = 1 and write_accept = 0 because ( wr_inc && !wr_full). - wr_ptr keep the position and value - wr_bin keep the value unchanged - wdata keep the value unchanged |
| 5 | Write | Write wrap around | - wr_ptr, wr_full, wr_bin | -Write 8 times (until wr_full = 1), then read 8 times (until rd_empty = 1) to drain the FIFO. - Repeat this write-then-read cycle for a total of 2 rounds (16 writes total), so that wr_bin (4-bit) completes one full wrap: 0000 → 0001 → ... → 1111 → 0000. | - wr_bin only advances on write_accept (wr_inc && !wr_full); not affected by rd_inc. - wr_bin wraps correctly: 0000 → ... → 1111 → 0000 after 16 writes. - wr_ptr (Gray) changes 1 bit per step, including at wrap, matching Table 4. - wr_full asserts/deasserts correctly each round, even across the wrap boundary. - No address collision when wr_bin's lower 3 bits repeat on the second pass. |
| 6 | Read | Single Read | - rd_ptr, rd_bin, rd_empty, rdata, read_accept, | - Put the rd_rst_n = 1 back and rd_inc = 1 when rd_empty = 0 . After that read the data out of the memory with the correct rd_bin | - read_accept = 1 ( rd_inc && !rd_empty). - rdata correct value that has been write in the address( rd_bin) - rd_bin increase 1 binary value, - rd_ptr increase 1 value ( gray code) - the data has been read out in the memory still keep the value and the value only change when overwrite it in wr_clk. |
| 7 | Read | Read from full untill empty | - rd_ptr, rd_bin, rd_empty, rdata, | - Put the write operation exact 8 times and wr_full = 1. - After that start : put rd_inc = 1 continuous for exact 8 times and check the rd_empty after the 8th read 1 clock cycles ( rd_empty should be = 1) | - rd_accept = 1 for continuous 8 times. - rd_bin go from 0000 to 1000 - rd_ptr go from 0000 to 1100 - rd_empty goes High ( = 1) after the 8th read operation 1 clock cycles. |
| 8 | Read | Read wrap around | - rd_ptr, rd_empty, rd_bin | - Write 8 times (until wr_full = 1), then read 8 times (until rd_empty = 1). - Repeat this write-then-read cycle for a total of 2 rounds (16 reads total), so that rd_bin (4-bit) completes one full wrap: 0000 → 0001 → ... → 1111 → 0000. | - rd_bin only advances on read_accept (rd_inc && !rd_empty); not affected by wr_inc. - rd_bin wraps correctly: 0000 → ... → 1111 → 0000 after 16 reads. - rd_ptr (Gray) changes 1 bit per step, including at wrap, matching Table 4. - rd_empty asserts/deasserts correctly each round, even across the wrap boundary. - rdata matches the originally written wdata, even after the address wraps and is reused. |
| 9 | Synchronizer | 2 FF latency ( put rd_ptr in wr_clk domain) | - rd_ptr , wq2_rdptr | - Put one read operations and check wq2_rdptr after 2 clock cycles in wr_clk domain | -- After 1 wr_clk edge following the rd_ptr change: wq2_rdptr still holds the OLD rd_ptr value (expected, not a bug). - After 2 wr_clk edges: wq2_rdptr equals the NEW rd_ptr value. - wq2_rdptr never shows X/unknown at any wr_clk edge throughout the test, not only at the 2nd cycle. |
| 10 | Synchronizer | 2 FF latency ( put wr_ptr in rd_clk domain) | - wr_ptr, rq2_wrptr | - Put one write operations and check rq2_wrptr after 2 clock cycles ( because of 2 FF) in rd_clk. | -- After 1 rd_clk edge following the rd_ptr change: rq2_wrptr still holds the OLD wr_ptr value (expected, not a bug). - After 2 rd_clk edges: rq2_wrptr equals the NEW rd_ptr value. - rq2_wrptr never shows X/unknown at any rd_clk edge throughout the test, not only at the 2nd cycle. |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |

s